# Preflight

## Interior

- Mags - Off

- Cowl flaps - Open

- Fuel selector - Left tank

- Gust lock - Remove

- Landing gear switch - Down

- Master switch - On

- Fuel gages - Check quantity

- Flaps - Down

- Master switch - Off

- Aircraft documents - ARROW

- Weight and balance - Review

- Flashlights/equipment - Check

## Exterior

- Empennage - Check security, movement, trim tab security/cable attachment free play

- Tail tiedown - Remove

- Right static port - Check unobstructed

- Underside - Check condition, antennas

- Right flap - Check for damage, security

- Right aileron - Check for damage, security, movement

- Right wing - Check for damage

- Right tiedown - Remove

- Right gear - Check strut (2 inches showing), uplock roller, brakes, and tire (30 psi)

- Right fuel tanks -

- Main gear - Check strut (2 inches showing) and tire (30 psi)

# Preflight (cont’d)

- Right cowl - Check secure

- Cowl plugs - Remove

- Propeller and spinner - Check condition

- Engine air inlet - Check unobstructed

- Left cowl - Open

- Alternator belt - Check tension

- Oil level - Check (9 quarts max. 8 quarts min.)

- Oil filler cap - Check secure

- Left cowl - Close

- Fuel selector drain - Check sump

- Left wing - Check for damage

- Left fuel tanks - Check quantity, vents, sump

- Left gear -.Check strut (2 inches showing), uplock roller, brakes, and tire (30 psi)

- Pitot tube - Remove cover. Check for obstructions

- Stall warning switch - Check movement, operation

- Navigation lights (night flight) - Check

- Left tiedown - Remove

- Left aileron - Check for damage, security, movement

- Left flap - Check for damage, security

- Left static port - Check unobstructed

# Starting Engine

## Cold

- Brakes - Set

- Landing gear switch - DOWN

- Circuit breakers - Check

- Fuel - Desired tank (Left?, NEVER Aux)

- Prop - Full increase

- Mixture - Rich

- Throttle - Open 2 turns

- Master switch - ON

- Fuel pump - LOW, wait until indication on fuel flow meter, then OFF

- Prop area - ``Clear!’’

- Starter - Engage

- Engine speed - 1000 RPM

- Fuel pump - OFF

- Oil Pressure - must indicate within 30 seconds

- Flaps - UP

- Flap switch - Center

- Beacon - ON

## Hot

Same as cold, except...

- Mixture - Rich

- Throttle - Open 1 turn

- Fuel pump - LOW - peak twice, then OFF

- Throttle - Open 3/4 of full throttle

- Prop area - ``Clear!’’

- Starter - Engage

- Fuel pump - LOW momentarily as engine fires

- Throttle - Reduce immediately

# After Start

- Nav lights - On, as appropriate

- Radio master - ON

- Transponder - STANDBY

- ATIS, Ground, Tower - SET                   (SQL 125.9/121.6/119.0)

- Altimeter - Set (within 40’ field elev.)

# Taxi

- Brakes - Test

- Artificial Attitude - Check operation

- Turn Coordinator - Check operation

- Directional Gyro - Check operation

- Magnetic compass - Check operation, fluid level

# Before Takeoff

## Engine Runup

- Brakes - Set

- Fuel - Desired tank (left?). NEVER aux!

- Mixture - Rich

- Oil temperature - Wait for 70°F

- Engine speed - 1800 RPM

- Mags - Test: L-Both-R-Both. Drop-off < 100 Rpm and < 50 RPM differential..

- Prop - Cycle twice full travel. Check oil pressure/man pres/RPM drop, normal return.

- Vacuum - Between 4 - 7 inches

- Alternator - Charging - show discharge

## Instruments

- Engine gages - Check

- Instruments - Check and set

- Mags - BOTH

- Prop - Full increase

- Mixture - Rich

- Elevator trim - 3° (green/white transition)

- Aileron trim- Neutral

- Flaps - 0° (first line 10° short/soft field)

- Flight controls - Free, correct, and smooth.

- Door, shout, and emergency windows - Latched

- Seat belts - Fastened

- Passengers - Briefed

## Navigation

- Tower/Departure freq. - Set

- Sqawk - Set

- Departure route/Navaid - Set

# Takeoff

- Transponder - ALT

- Mixture - Rich

- Prop - Full increase

- Strobes - ON

- Landing lights - As appropriate

- HSI - Check heading with runway number

- Departure time - Record

- Mixture - Lean for takeoff, if elevation > 3000 feet

- Rotate - 60-70 mph, VX - 75 mph Vy - 103 mph (sea level)

# Post Takeoff

- Gear - UP (when out of usable runway)

- Flaps - Up

- Throttle/Prop - 1000 feet AGL, - 24’’/2400 RPM

# Climb

- Each cardinal 1000 feet - Adjust manifold pressure

- Pass 3000/5000/7000 feet - Adjust mixture

# Cruse

- Throttle/Prop - Set for cruse

- Mixture - Lean 50°F (25° if 65% power) below peak.

- Cowl flaps - Close

# Landing - Downwind

- Gas Fuel - Proper tank (NOT aux)

- Undercarriage Gear - DOWN (140 mph max)

- Mixture - Full rich (sea level) or as appropriate

- Prop - Full increase (after power reduction)

- Switches/Seatbelts - Landing lights/Passengers briefed and belted

- Flaps - Set (120 mph max)

- Approach speed - 90 mph

# Landing - Final

- Prop - Full increase

- Cowl flaps - Open

- Gear - DOWN indicated locked

- Speed over the fence - 80 mph

# Go Around

- Prop - Full increase

- Throttle - Full power

- Mixture - Rich (sea level) or as appropriate

- Throttle - Full power

- Flaps - Up gradually

- Gear - UP

- Cowl flaps - Open

- Throttle/Prop - 1000 feet AGL - 24’’/2400 RPM

- VX - 75 mph Vy - 103 mph

# After Landing

- Flaps - Raised

- Transponder - STANDBY

- Strobes - OFF

# Shutdown

- Radio master - OFF

- Electrical Equipment - OFF

- Throttle - 1000 RPM

- Prop - Full increase

- Mixture - Idle cut-off

- Master switch - OFF

- Mags - OFF

- Arrival Time - Record

# Secure

- Flight time - Record in airplane log book

- Elevator trim - 3° (green/white transition)

- Cowl flaps - Close

- Gust lock and bungee cords - Install

- Instrument panel - Cover with towel

- Cabin door - Close and lock

- Airplane cover - Install

- Pitot tube cover - Install

- Cowl plugs - Install

- Tail and wing tiedowns - Secure

- Flight plan - Close

# EMERGENCY Gear Extension

- Master switch - ON

- Gear - DOWN

- Gear indicator - Check green lamp lit or lower indicator in the DOWN position.

If gear down indication still not present:

- Airspeed - < 120 mph

- Gear circuit breaker - Pull

- Gear crank - approx. 50 counterclockwise turns

- Lower gear indicator - Verify in down position

- Gear crank - Stow

Note: with landing gear circuit breaker “pulled”, landing gear position lights are nonfunctional.

# EMERGENCY Gear Up Landing

- In an emergency, a gear up landing may be made:

- When the surface is too soft or rough to permit a satisfactory gear down landing.

- When a field is too short for a gear down landing, one that might cause more damage by hitting obstructions than a gear up landing would.

- When a water landing is necessary.

- Flaps - UP

- Throttle - CLOSED

- Master - OFF

- Mags - OFF

- Fuel selector - OFF

# EMERGENCY Power Loss

- Best glide speed - 103 mph

- Select suitable landing area

- Fuel selector - Switch to tank not in use

- Throttle - Open 1/2 inch from present setting

- Prop - Full increase

- Fuel pressure - Check. If none, Fuel pump - HIGH

- Mags - Check best: left, right, both.

- To restart failed engine:

- Fuel pump - OFF

- Throttle - Closed

- Mixture - Idle cut-off

- Fuel selector - OFF

- Wait several seconds

- Fuel elector - Fullest tank

- Mixture - Rich

- Throttle - Open 1/4 inch

- Fuel pressure - If zero - Fuel pump - HIGH

- If engine cannot be restarted:

- Mixture - Idle cut-off

- Throttle - Closed

- Mags - OFF

- Fuel selector - OFF

- Prop - Low RPM

- Best glide speed - 90 mph

# EMERGENCY Engine Fire In Flight

- Mixture - Idle Cut-off

- Fuel selector - OFF

- Mags - OFF

- Throttle - Closed

Best glide speed - 103 mph